Basic Guidelines for FPV Camera Setup

- Note: I’ll make this guide more step-by-step and detailed once I have the quad itself and some of the parts required to mount the camera so I can understand it physically. For now, I came up with this basic outline of stuff we need to get.

- Main source used for this document: https://forum.bitcraze.io/viewtopic.php?f=6&t=491

- To Buy: (total price for everything to mount the camera on the quad including shipping should be around $200)

  - Camera

    - Price: $54.00

    - PAL system 762×572 pixels

    - Comes with cable

    - Weight: 1 gram

    - Has UV+IR filter lens for daylight and non-filtered lens for night vision

  - Transmitter

    - Price: $29.00

    - Size: 20x20x4mm

    - 10mw, 5.8gHz, Audio Video Transmitter module

    - Weight: 1.2 g

    - Current: 70mA

    - 8 channels

  - OR here’s and even smaller 2.4gHz nano video transmitter for $34.99

  - 5V Step-Up Voltage Regulator

    - Price: $3.95

    - Boosts voltage as low as 2.5 V to 5 V

  - Lens

    - Price: $14.90

    - Weight: 1.5 g

  - Receiver

    - Price: $26.90

    - 5.8gHz A/V Receiver Module

    - Sensitivity: -90dBm

    - Size: 28x21x3mm

    - Voltage: 3.2v - 5.5v

    - Current: Current: 170 mah

    - RFin: 50 Ohm

  - Digitizer

    - Price: > $49.67

    - Hauppauge 610 USB-Live 2 Analog Video Digitizer and Video Capture Device

    - To watch, capture, or record live video

  - Camera Holder

    - Price: $4.04

    - 3-D Printed

- Connections:

  - Flie Pin 17 DGNG wired to

    - Regulator GND

    - TX GND

  - Flie Pin 16 VCOM wired to a slide switch wired to

    - Regulator VIN

  - Regulator VOUT wired to

    - Cam VIN

    - TX VIN

  - TX GND_cam and TX VID wire to Cam GND and Can VID respectively

- Software:

  - Flie driver node

    - Written in python. Communicates with the flie. Advertises crazyflie logs as ROS messages, changes parameters on flie and forwards control commands.

  - Dynamic reconfigure

    - Interface to change flie params (http://ros.org/wiki/dynamic_reconfigure)

  - Flie control node

    - Listens to joystick messages and converts them into flie commands; has different settings (eg sensitivity, etc)

  - C++ GUI mode

    - Listens to logs and camera data, uses QGL Widget to draw everything

  - usb_cam node

    - outputs digitized image (http://www.ros.org/wiki/usb_cam)

  - ps3joy

    - Bluetooth driver for sony gamepad (http://www.ros.org/wiki/ps3joy)